# ESP Lab1: Short Report

# Signal Modulation by Decimation and Interpolation

| Group / Team | Team member 1: | First, Last name (please change) |
| --- | --- | --- |
| ESPL05 / F (please change) | Team member 2: | First, Last name (please change) |
|  | Team member 3: | First, Last name (please change) |

Table 1: Different configurations to be used by different lab groups

| Lab group as chosen in myHAW | Down- / Upsampling factor | Chirp, from … to [kHz], length [s] |  | Bat sound fbat | Sampling frequency ADC, DAC fs [kHz] | Polyphases xλ output at DAC CH0 / CH1 |
| --- | --- | --- | --- | --- | --- | --- |
| ESPL/01 | M=L=3 | 18 … 21 kHz | 2.5 s | 18 kHz | 50 kHz, 50 kHz | λ=0 / λ=2 |
| ESPL/02 | M=L=4 | 14 … 17 kHz | 3.0 s | 15 kHz | 50 kHz, 50 kHz | λ=0 / λ=3 |
| ESPL/03 | M=L=5 | 21 … 24 kHz | 2.0 s | 17 kHz | 50 kHz, 50 kHz | λ=0 / λ=4 |
| ESPL/04 | M=L=5 | 18 … 21 kHz | 3.0 s | 16 kHz | 50 kHz, 50 kHz | λ=0 / λ=3 |

# Lab task

## Echo program

Take a screenshot of the amplitude response and attach it to the lab report. In the report, describe the shape of the frequency response and explain how this affects signal transmission in the DSP system.

## Implementation of multirate system

After successful testing, insert the final C code of adcInt() and dacInt() into the lab report. Apply a sine wave of frequency fbat as specified in Table 1 for your group to both inputs of the system and measure the amplitude spectrum via FFT at the output. Take a screenshot for the report. Increase the frequency by 5 kHz and compare the amplitude spectrum to the previous. Explain your observations.

## Insert the interpolation filter

Compare amplitude spectra and time-domain signals of both DAC channels 0 and 1 again for the bat frequency fbat as given in the Table 1. Add relevant screenshots to your report and explain the results.

## Optional task: let the bats fly

A wave file containing bat sounds has been stored on the internship computer. These are not the original sounds, which are in the 40-50 kHz range, but were shifted to the upper human hearing range. Input the signal to your system and compare the animal sounds at input and output of the multirate system and with and without interpolation filters by listening and using the audio analyzer.

Congratulations! You made it. Please submit with 15 min. after end of the session.